# ADD FOR A SPORTS CLUB-DSD FIRST YEAR PROJECT

Braica Patricia

Project supervisor: Timar Mihai

- SPECIFICATIONS

You are required to make an ad with multiple animations. The seven segment display will be used for this purpose. The text will consist of symbols of an available alphabet. The ad will have multiple operating modes (minimum 4) that can be selected by the user from the switches of the FPGA.  The quartz oscillator imbedded in the board will be used, it’s frequency will be, of course, divided. Because you cannot represent all letter using the SSD, a maximal alphabet will be created.

The finished product will have 8 different animations for the same text and you will be able to switch the message without difficulty.

- DESIGN

Inputs:

- Clock-100 MHz

- RESET-when pressed nothing is shown

- Select-selects one of the 8 animations

  - BLACK BOX

Outputs:

- Segment-the binary codification of a symbol that appears on the SSD at a given moment

- Select-the binary representation of the anods(when anods(n)=1 the n+1’th display will be lit since we start from anods(0))

  - CONTROL AND EXECUTION UNIT

The block diagram was made so it would correspond to the state diagram

The memories used for the project are:

The maximal alphabet encoded      The given_word memory(uses the alphabet  memory)

We declare  them in the main entity’s architecture with their types as:

type mem is array (26 downto 0) of std_logic_vector(6 downto 0);
type word is array (20 downto 0) of std_logic_vector(6 downto 0);

We use two counters to divide the frequency:

The first is used for the actual display and matches the refreshing rate of the images. The human’s eye has 16ms image refreshing rate.

So we use a 16 bit counter (15 downto 0) and choose an approximating value.

entity counter15 is
port(
clk :in std_logic;
o: out std_logic); 
end counter15;
Architecture count2 of counter15 is
signal c: std_logic_vector(15 downto 0):=(others=>'0');
begin

counting : process(clk)
begin
if(clk='1' and clk'event) then
 c<=c+1;
if(c="1111000000000000") then
o<='1';
end if;
if(c="1111000000000001") then
c<=(others=>'0');
o<='0';
end if;
end if;

end process counting;

end count2;

When the counter fills up to that value the signal count2 is transmitted to the for_display process.

The second counter is used to toggle the changing of frames. I use a big value for the counter approximatively 5000000, half of the actual frequency of the clock. So the execution of the program will be delayed with a half of a second, that is, a new frame will appear half of a second after the previous one.

entity counter25 is
port(
clk :in std_logic;
o: out std_logic); 
end counter25;
Architecture count1 of counter25 is
signal c: std_logic_vector(25 downto 0):=(others=>'0');
begin

counting : process(clk)
begin
if(clk='1' and clk'event) then
c<=c+1;
if(c="10111110101111000010111111") then
o<='1';
end if;
if(c="10111110101111000011000000") then
c<=(others=>'0');
o<='0';
end if;
end if;

end process counting;

end count1;

The output of the counter is transmitted to the frame changing process.

Every register is changed based on the current value so, in addition to the drawing, there is a wire that goes from each register to the Multiplexors block . The current value will be used as the select of the future value multiplexor. The SELECT acts like an enable for the multiplexors because, depending on the SELECT only certain registers change:

Here the SELECT is “011” and only segment4 changes

Here the SELECT is “110” and only s78, c78 and bitmask8 change

COMPLETE DESIGN FOR THE SEGMENT4 REGISTER

The same concept is applied for all registers in the frame change process

This multiplexer does the same operations as a shift register would, the anod goes from 0 to 7 and than this process is repeated

if(anod(0)='1') then
            anod(0):='0'; 
            anod(1):='1';
        elsif (anod(1)='1') then 
            anod(1):='0';
            anod(2):='1';
        elsif (anod(2)='1') then 
            anod(2):='0';
            anod(3):='1';
        elsif (anod(3)='1') then 
            anod(3):='0';
            anod(4):='1';
        elsif (anod(4)='1') then 
            anod(4):='0';  
            anod(5):='1';
        elsif (anod(5)='1') then 
            anod(5):='0';
            anod(6):='1';
        elsif (anod(6)='1') then 
            anod(6):='0';
            anod(7):='1';
        elsif (anod(7)='1') then 
            anod(7):='0';
            anod(0):='1';            
                                    end if;
                                                       anods<=anod;

The register j represents the address in memory to be prepared from which the SSD encoding is chosen. It will take the value of the index of the next active anod(if the anod(0) is lit, anod(1) follows so j=1).

case anod is
when "00000001"=>
j:=1;
when "00000010"=>
j:=2;
when "00000100"=>
j:=3;
when "00001000"=>
j:=4;
when "00010000"=>
j:=5;
when "00100000"=>
j:=6;
when "01000000"=>
j:=7;
when "10000000"=>
j:=0;
when others=>
j:=1;
end case;

  - ANIMATIONS

The left blocks of code are from the fordisplay process and the right from framechanging process.

- Animation 1 -Clipping

This animation consists of two frames. The word on the 8 SSDs will be displayed and then disappear.

clipping<=clipping+1;
if(clipping=2) then
clipping<=0;
end if;

We will use a variable clipping that will take either 1 or 0 and will change its value in the frame changing process. When clipping is 0 the word will show and when is 1 will disappear.

if(clipping=0) then	
segment<=given_word(j);
else
segment<=(others=>'0');
end if;

- Animation 2 –Filling from the outside to the inside and backwards

We will use two variables l2 and r2 that begin with the values 0 and 7 and at each frame change their values either approach or move away with 2 positions. The variable close2 will indicate which way their values change.

if(close2=1) then
l2<=l2+1;
r2<=r2-1;
if(l2>r2) then
l2<=l2-2;
r2<=r2+2;
close2<=0;
end if;

elsif(close2=0) then
l2<=l2-1;
r2<=r2+1;
if(l2<0 and r2>7) then
l2<=l2+1;
r2<=r2-1;
close2<=1;
end if;

end if;

if(j<=l2 or j>=r2) then
segment<=given_word(j);
else
segment<=(others=>'0');
end if;

- Animation 3 – Letters appear in a particular order

We will use the variable segment3 of size 8 and it is used in this way:

case segment3 is
when "01000000"=>
segment3<="01000010";
when "01000010"=>
segment3<="01001010";
when "01001010"=>
segment3<="01011010";
when "01011010"=>
segment3<="11011010";
when "11011010"=>
segment3<="11011110";
when "11011110"=>
segment3<="11111110";
when "11111110"=>
segment3<="11111111";
when "11111111"=>
segment3<="00000000";
when "00000000"=>
segment3<="01000000";
when others=>
segment3<="00000000";
end case;

If the segment3(x) is 1 the letter will be displayed and if 0  it won’t.

if(segment3(j)='1') then
segment<=given_word(j);
else
segment<=(others=>'0');
end if;

- Animation 4 – Two letters disappear from the word at a time

case segment4 is
when "11111111"=>
segment4<="11101110";
when "11101110"=>--
segment4<="11011101";
when "11011101"=>
segment4<="10111011";
when "10111011"=>
segment4<="01110111";--
when "01110111"=>
segment4<="11111111";
when others=>
segment4<="11111111";
end case;

To be explicit the first and fifth letter, the second and the sixth ,  …, the fourth and the last. The difference between them is 4. We will use just like in the previous animation a variable segment4.

if(segment4(j)='1') then
segment<=given_word(j);
else
segment<=(others=>'0');
end if;

- Animation 5 – Right to left display

i5<=i5+1;
if(i5>j56) then
i5<=-7;
end if;

We will have the variables: j56- which is the length of the word-1, so the index of the last symbol and i5=-7 and it’ s value increments every frame until it reaches j56. If the i5+x value is between 0 and j56 the symbol at that position will be displayed.

j:=j+i5;
if(j>=0 and j<=j56) then
segment<=given_word(j);

else
segment<="0000000";
end if;

- Animation 6- Left to right display

i6<=i6-1;
if(i6<-7) then
i6<=j56;
end if;

We use the same ideea, the only difference is that i6 decrements at each frame change

j:=j+i6;
if(j<=j56 and j>=0) then
segment<=given_word(j);
else
segment<="0000000";
end if;

- Animation 7- Flowing from left to right

We will use a bitmask  that can take 3 values selected using a multiplexer for the three states of the flow animation and use the and operation on the bits. Variable s78 tells which sides of the segment will be turned on(just like in the figure) and c78 tells what letter we have reached in our animation.

s78<=s78+1;
if(s78>2) then
s78<=0;
c78<=c78+1;
if(c78>7) then
c78<=0;
end if;
end if;
case s78 is
when 0 =>
bitmask7<="0010010";
when 1=>
bitmask7<="0110111";
when 2=>
bitmask7<="1111111";
when others=>
bitmask7<="0000000";
end case;

if(j<c78) then
            segment<=given_word(j);
            elsif (j=c78) then
               segment<=given_word(j) and bitmask7;
            else  
            segment<="0000000";
            end if;

- Animation 8- Flowing from top to bottom

if(j<c78) then
            segment<=given_word(j);
            elsif (j=c78) then
              segment<=given_word(j) and bitmask8;
            else  
            segment<="0000000";
            end if;

s78<=s78+1;
if(s78>4) then
s78<=0;
c78<=c78+1;
if(c78>7) then
c78<=0;
end if;
end if;
 case s78 is
 when 0 =>
 bitmask8<="0000100";
 when 1=>
 bitmask8<="0001110";
  when 2=>
  bitmask8<="0001111";
  when 3=>
  bitmask8<="1011111";
  when 4=>
  bitmask8<="1111111";
  when others=>
  bitmask8<="0000000";              
 end case;

The principle is the same as the previous one, the difference is that we have 5 states( just like in the picture).

  - How the project functions on the FPGA board

I used the application LOGISIM EVOLUTION in order to put the project on the board.

What the main window contains with the project anotations:

Input_1 represents the clock signal and

is mapped on the red area where the FPGA in-built clock resides.

Input_2 represents the reset signal and it is used in order to reset the animations or change the animation. It is mapped on the red coloured switch

Input_bus_1 represents the select signal which is 3 bits-long and I mapped them on the red switches in the following way from left to right:

Input_bus_1#2 - Input_bus_1#1 -  Input_bus_1#0

Output_bus_1 represents the binary encoding signal on the cathode of the symbol at a given time and I used the encoding mapping given in the following figure. So, for example, the Output_bus_1#0 will be mapped on the pink line, Output_bus_1#1 on the blue line and so on.

Output_bus_2 represents the array of the anodes

And it is mapped on the red areas in the following way from left to right:

Output_bus_2#0, Output_bus_2#1, Output_bus_2#2, … , Output_bus_2#7

Functioning regime

The moment the project will be loaded on the board the animation will be playing. In order to change the animation you need to flip the reset switch and only then change the select switches and flip the reset again in order to avoid any errors.

  - References

https://sites.google.com/site/learnvhdl/home/hour-15-state-machine-contd

https://digilent.com/reference/programmable-logic/nexys-4-ddr/reference-manual

https://surf-vhdl.com/vhdl-array/

https://cse.usf.edu/~haozheng/teach/cda4253/doc/vhdl-stmt.pdf